APX6000

Scan On/Off  - Top Side Button

- Press and release the Top Side Button to turn scan on or off.

You see the current scan state – SCAN ON or SCAN OFF. The scan status symbol   is displayed, indicating scan is active.

Edit the Scan List -  Menu

- Press the right side of the 4-way navigation button until SCLN is displayed.

- Press and release the menu button below SCNL.

- Programming icon appears on the display.

- Select the Zone by depressing the right or left navigation button and the 16 button select knob to select the channel to be scanned.

- Press the button directly below SEL or DEL or RCL.

- Repeat steps for each channel to be scanned

- Press the HOME button to exit editing Scan List.

Keypad Tones Mute/Unmute (Menu)

- Press the button directly below MUTE.
The current state is shown – TONES OFF or TONES ON.

- Press the button directly below OFF or ON.

Note: Press the Home or the PTT button to exit without changing the setting. Turning off keypad tones does not affect any other tones.

#### Keypad Lock/Unlock – 2-Position Concentric Switch

Keypad Lock – Move 2-Position Concentric Switch to .

Keypad Unlock -  Move 2-Position Concentric Switch to O.

Quick

Reference

Card

| Tones Chart |  |
| --- | --- |
| Symptom | Possible Cause/Solution |
| Busy Tones A Group of Low-Pitched Tones | You cannot transmit because all channels are busy, release PTT and wait for call-back. |
| Three short high pitched tones | Talk Permit. You can now continue with your requested transmission. |
| Continuous low-pitched tone when pressing the PTT button | Indicates you are out of range, or the Time-Out Timer has activated. May also indicate an Invalid Mode, or Talk Prohibit/PTT Inhibit. |

| Backlight and Bar Color | Notification | When |
| --- | --- | --- |
| Orange | Emergency Alerts | The radio initiates an emergency alarm or call. |
|  |  | The radio receives an emergency alarm or call. |
| Red | Critical Alerts | The radio is out of range. |
|  |  | The radio enters failsoft mode. |
|  |  | The radio is unable to establish a full connection with the system. |
| Green | Call Alerts | The radio receives a private call. |
|  |  | The radio receives a call alert. |

| Turn Radio On/Off - Turn the radio on by rotating the On/Off/Volume Control Knob clockwise until you hear a click. - Turn the radio off by rotating the On/Off/Volume Control Knob counterclockwise until you hear a click. Select a Channel - Rotate the 16-position Channel select knob to the desired Channel. The new channel name will appear on the display. - Press the PTT button to transmit. When speaking, keep the microphone 1-2” from your mouth. - Release PTT button to listen. Select a Zone - Menu - Press the right side of the 4-way navigation button until ZONE is displayed. - Press the right side of the 4-way navigation button until the desired zone is displayed. - Press the Home button to confirm the displayed Zone. - Select a Zone – Side Button 1 or 2 - Press Side button 1 to Zone Up or Side button 2 to Zone Down. #### Emergency – Trunked Feature (Top Orange Button) - With the radio turned on, press the emergency button. - The LED lights solid red, the display alternates between showing “EMERGENCY” and the current trunked zone and talkgroup/channel, and a short, medium-pitched emergency tone sounds. - 2. Press the PTT button and announce your emergency. - 3. Release the PTT button to end the transmission and wait for a response from the dispatcher. - To exit emergency: Press and hold the Emergency button for about one second. - The radio user should NEVER clear the emergency on their radio without being instructed to do so by dispatch. | Mode Select -Saving a Zone and Channel to keypad Save a zone and channel to a keypad digit. Procedure: 1. Toggle your zone and channel to the required zone and channel. 2. Press and hold (long press) the keypad digit button you desire to program. You hear a short, medium-pitched tone when the zone and channel is saved. To switch the radio to the programmed Zone and Channel: - Press and release (short press) the keypad digit. Respond to a Private Conversation Call When a Private Conversation Call is received, two alert tones are heard, the LED blinks green, the Call Received symbol () flashes, and “CALL RECEIVD” is displayed and menu select item will automatically display RESP. - Press the menu select button below RESP button within 20 seconds after the call indicators begin. - Press and hold the PTT button to talk. Release the PTT button to listen. - Press the Home to hang up and return to the Home screen. Respond to a Call Alert Page When you receive a call alert page you hear four repeating alert tones and the LED blinks green. The call received icon blinks and display shows “PAGE RECEIVED”. - Press PTT or any button to clear the page. |
| --- | --- |

Top  View

APX6000 Controls

Initiate/Cancel an Emergency Alarm

- To initiate an alarm, press the Emergency button.  An alert tone (beep) sounds and the display alternates between EMERGENCY and the selected talkgroup and the emergency is transmitted.

- A single beep indicates acknowledgement by the central controller (not necessarily the dispatcher).

- After a dispatcher acknowledgement the radio will beep 4 times and the display will read ACK RECEIVED.

- To cancel an alarm, press and hold the Emergency button until you hear a continuous exit tone.

View Your ID Number

- Press the Call or Page button.

- Press the Mode Select button (backward once) to display “MY ID: ######” with the unit’s ID number.

- You can also press the “*” button to quickly display the radio’s ID number.

- Press the Call or Home button to exit.

* Buttons may be redefined in programming. Refer to your specific radio program. Print Date: 07/31/26

16-Position

Select Knob

Emergency

3-Position A/B/C

Not Programmed

On/Off/Volume

Control Knob

2-Position Concentric

Keypad Lock/Unlock

Top Side

Light/Flip

Push-to-Talk

(PTT) Button

Side Button 1

Site Display

Side Button 2

Keypad Lock

Side View

Menu Select Items

Nuis    Prfl      Zone
ScnL    Mute  Batt

Info     Rcnt    Cnts
Myid

Menu Select

Buttons